ELEVATOR PREDICTIVE MAINTENANCE

Full Production Implementation Plan

| Project | AI Health Prediction & Camera Monitoring — Phase 1 |
| --- | --- |
| Document | Full Production Implementation Plan |
| Sensors | ES-VS-01 · ES35-SW · RW-ST01D + HD-MV01A |
| Target | Production-ready by end of August 2025 |
| Classification | Internal — Confidential |

# 0. Sensor Stack Summary

All four sensors communicate via RS-485 Modbus RTU on a shared daisy-chain bus. Each sensor has a unique Slave ID and can be polled independently from a single edge device UART port via a USB→RS485 converter.

| Sensor ID | Type | Protocol | Poll Rate | Key Outputs | Baud Rate | AI Usage |
| --- | --- | --- | --- | --- | --- | --- |
| ES-VS-01 | Vibration 3-axis | Modbus RTU | 1–10s | Accel RMS, Velocity RMS, Peak, Temp | 9600 | Anomaly detection, motor health trending |
| ES35-SW | Temp + Humidity (SHT35) | Modbus RTU | 10–30s | Temperature °C, Humidity %RH | 9600 | Environment monitoring, motor overheating |
| RW-ST01D | Loadcell converter (RS232→RS485) | Modbus RTU | 1–5s | Weight kg / N (from HD-MV01A) | 9600 | Cabin load monitoring, overload detection |
| HD-MV01A | Load cell (analog) | Analog → RW-ST01D | via converter | Raw strain gauge mV/V | — | Raw weight signal — processed by RW-ST01D |

⚠️ Important: ES-VS-01 outputs pre-processed RMS values — NOT raw waveform The ES-VS-01 computes RMS/Velocity/Peak internally using its MEMS accelerometer. You receive aggregated values per poll, not raw time-series samples at kHz. This means: - FFT-based bearing fault detection is NOT possible with this sensor alone - You ARE able to do: trend monitoring, threshold alerting, time-series anomaly detection (LSTM/Isolation Forest), RUL estimation via degradation curves - Poll interval recommendation: 5 seconds for vibration during operation, 30 seconds for temperature/humidity

# 1. Architecture Decisions

## 1.1  Deployment Model — Edge-First

All inference runs on an edge device co-located with the elevator controller panel. Cloud is used only for dashboarding, alerting, and long-term storage. This decision is driven by:

- Real-time alerting requirement < 3 seconds — cloud round-trip adds latency

- Elevator shafts often have poor network connectivity

- Data sovereignty — raw sensor data stays on-premises

- Hardware budget constraint — no cloud GPU needed

| Component | Option Chosen | Alternatives Rejected | Reason |
| --- | --- | --- | --- |
| Edge hardware | Orange Pi 4 Pro | Jetson Nano, x86 mini-PC | RK3399 hexa-core CPU, 4GB LPDDR4, USB 3.0, M.2 slot, good Linux support, sufficient for ONNX CPU inference |
| OS | Ubuntu 20.04 LTS | Raspberry Pi OS, Windows IoT, Armbian | Stable, well-documented for ML workloads |
| Language | Python 3.11 | Go, C++ | Fastest iteration, rich ML/sensor libraries |
| Serial driver | minimalmodbus + pyserial | pymodbus, custom UART | Lightweight, simple, no broker needed |
| API framework | FastAPI + uvicorn | Flask, Django | Async-native, auto Swagger docs, Pydantic validation |
| ML runtime | ONNX Runtime (CPU) | TFLite, PyTorch | Framework-agnostic, fast CPU inference |
| Database (edge) | SQLite via SQLAlchemy | PostgreSQL, InfluxDB | Zero-config, embedded, sufficient for PoC |
| Database (cloud) | PostgreSQL (RDS/Supabase) | MongoDB, TimescaleDB | Relational, good for structured sensor + event data |
| Dashboard | Streamlit (PoC) → React (prod) | Grafana, Tableau | Fast to build; React for production UX |
| Alerting | Slack webhook + Email (SMTP) | PagerDuty, OpsGenie | Zero cost, sufficient for PoC and small fleet |
| Containerization | Docker + docker-compose | Kubernetes, bare-metal | Single-node deployment, simple to manage |
| CI/CD | GitHub Actions | Jenkins, GitLab CI | Free tier, integrates with GitHub repo |

## 1.2  Data Flow Architecture

SENSOR LAYER ES-VS-01 (vibration) + ES35-SW (temp/humidity) + RW-ST01D+HD-MV01A (load) → RS-485 bus → USB-RS485 adapter ▼ EDGE DEVICE — Orange Pi 4 Pro sensor_poller.py (Modbus RTU, pyserial) → Redis queue → preprocessor.py → ONNX inference → rule_engine.py → SQLite log ▼ LOCAL API FastAPI server → REST endpoints → Dashboard (Streamlit) + Alert dispatcher (Slack/SMTP) ▼ CLOUD SYNC (async, non-blocking) Background job → sync SQLite rows → PostgreSQL (cloud) → production dashboard (React) + long-term analytics

## 1.3  ML Model Architecture

| Model | Input Features | Output | Algorithm | Trigger |
| --- | --- | --- | --- | --- |
| Vibration Anomaly | accel_rms, velocity_rms, peak_accel (rolling 10min stats) | NORMAL / WARNING / CRITICAL | Isolation Forest → XGBoost | Every poll (5s) |
| Motor Health Score | vibration + temp + load (multivariate) | 0–100 health score | LSTM Autoencoder | Every 1 min |
| Overload Detection | load_kg vs max_capacity | SAFE / OVERLOAD | Rule-based threshold | Every poll (1s) |
| RUL Estimation | health_score trend (7-day window) | Hours to maintenance | Linear regression on degradation curve | Daily batch job |

# 2. Backend Services

## 2.1  Service Map

| Service | File | Runs on | Responsibility |
| --- | --- | --- | --- |
| sensor_poller | sensor_poller.py | Edge (always on) | Poll all 3 Modbus sensors on schedule, write raw readings to SQLite + Redis queue |
| preprocessor | preprocessor.py | Edge (always on) | Consume Redis queue, compute rolling features (RMS delta, rate-of-change, z-score), write feature vectors to DB |
| inference_engine | inference_engine.py | Edge (always on) | Load ONNX models at startup, run anomaly + health score inference per new feature vector |
| rule_engine | rule_engine.py | Edge (always on) | Apply threshold rules on inference output, trigger alerts, log events |
| alert_dispatcher | alert_dispatcher.py | Edge (always on) | Send Slack webhook / SMTP email, rate-limit duplicate alerts (1 per 15 min per elevator per type) |
| api_server | main.py (FastAPI) | Edge + Cloud | Expose REST + WebSocket endpoints for dashboard and external consumers |
| cloud_sync | cloud_sync.py | Edge (background) | Periodic batch upload of sensor_readings + events to cloud PostgreSQL |
| rul_scheduler | rul_job.py | Edge (cron daily) | Run RUL estimation on 7-day health score trends, write maintenance schedule |
| model_trainer | train.py | Offline / Cloud VM | Retrain models when labeled data grows, export ONNX, update registry |

## 2.2  sensor_poller.py — Full Implementation

Design principles - Non-blocking: each sensor polled in its own thread with individual try/except - Configurable poll intervals per sensor type via config.yaml - Exponential backoff on repeated Modbus read errors (max 60s) - Write raw readings to SQLite immediately — never lose data even if downstream fails

# sensor_poller.py import minimalmodbus, threading, time, sqlite3, redis, yaml, logging from datetime import datetime class SensorPoller: """Thread-safe Modbus RTU poller for all elevator sensors""" def __init__(self, config_path='config.yaml'): self.cfg = yaml.safe_load(open(config_path)) self.bus = self._init_bus() # shared RS-485 port self.redis = redis.Redis() # local Redis self.db = sqlite3.connect('elevator.db', check_same_thread=False) def _init_bus(self): inst = minimalmodbus.Instrument( self.cfg['serial']['port'], # e.g. /dev/ttyUSB0 slaveaddress=1) # overridden per-read inst.serial.baudrate = 9600 inst.serial.timeout = 1.0 return inst def read_vibration(self): # ES-VS-01, Slave ID=1 self.bus.address = 1 return { 'sensor_id': 'ES-VS-01', 'accel_rms_mg': self.bus.read_register(0x00, 2), 'velocity_rms_mms': self.bus.read_register(0x01, 2), 'peak_accel_mg': self.bus.read_register(0x02, 2), 'temperature_c': self.bus.read_register(0x03, 1), 'timestamp': datetime.utcnow().isoformat() } def read_temp_humidity(self): # ES35-SW, Slave ID=2 self.bus.address = 2 return { 'sensor_id': 'ES35-SW', 'temperature_c': self.bus.read_register(0x00, 1) / 10.0, 'humidity_pct': self.bus.read_register(0x01, 1) / 10.0, 'timestamp': datetime.utcnow().isoformat() } def read_load(self): # RW-ST01D + HD-MV01A, Slave ID=3 self.bus.address = 3 raw = self.bus.read_registers(0x00, 2) # 32-bit float import struct kg = struct.unpack('>f', struct.pack('>HH', *raw))[0] return {'sensor_id':'RW-ST01D','load_kg':kg,'timestamp':datetime.utcnow().isoformat()} def poll_loop(self, sensor_fn, interval_s, elevator_id): backoff = 1 while True: try: data = sensor_fn() data['elevator_id'] = elevator_id self._write_db(data) self.redis.lpush('sensor_queue', json.dumps(data)) backoff = 1 except Exception as e: logging.error(f'Sensor error: {e}, backoff={backoff}s') time.sleep(min(backoff, 60)); backoff *= 2 continue time.sleep(interval_s)

⚠️  Register addresses (0x00, 0x01...) above are typical for these sensor classes. Verify against the actual datasheet / register map provided by EPCB for ES-VS-01 and RW-ST01D before deploying.

## 2.3  Feature Engineering — Preprocessor

The preprocessor consumes raw readings from Redis and computes rolling-window features before passing to the inference engine:

| Feature Name | Formula | Window | Source | Purpose |
| --- | --- | --- | --- | --- |
| accel_rms_mean | mean(accel_rms, W) | 10 min | ES-VS-01 | Baseline vibration level |
| accel_rms_std | std(accel_rms, W) | 10 min | ES-VS-01 | Stability / consistency |
| accel_delta | current - rolling_mean | 10 min | ES-VS-01 | Deviation from normal |
| accel_roc | (t - t-1) / interval | — | ES-VS-01 | Rate of change — sudden spikes |
| velocity_rms_z | (x - mean) / std | 1 hour | ES-VS-01 | Z-score: how many σ from normal |
| peak_to_rms_ratio | peak_accel / accel_rms | — | ES-VS-01 | Crest factor proxy — impulsive faults |
| motor_temp_delta | temp_vibsensor - temp_env | — | VS-01 & ES35-SW | Motor heat above ambient |
| humidity_trend | slope(humidity, W) | 30 min | ES35-SW | Rising humidity = moisture ingress risk |
| load_pct | load_kg / max_capacity | — | RW-ST01D | Capacity utilization |
| load_variance | std(load_kg, W) | 5 min | RW-ST01D | Jerk / irregular loading |
| multivariate_score | concat all features above | — | All sensors | Input to LSTM Autoencoder |

# 3. Database Schema

SQLite is used on the edge for real-time writes. PostgreSQL in the cloud stores the same schema for long-term analytics and multi-elevator fleet management. All tables use UTC timestamps.

## 3.1  Core Tables

### elevators

| Column | Type | Constraints | Description |
| --- | --- | --- | --- |
| id | TEXT | PK | UUID v4 — unique elevator identifier |
| name | TEXT | NOT NULL | Human-readable name, e.g. 'Tower A - Lift 3' |
| location | TEXT | NOT NULL | Building / floor / shaft description |
| max_capacity_kg | REAL | NOT NULL | Load cell max rated capacity |
| install_date | TEXT | NOT NULL | ISO date of installation |
| last_maintenance | TEXT |  | ISO datetime of last service |
| status | TEXT | DEFAULT 'active' | active ¦ decommissioned ¦ maintenance |
| created_at | TEXT | NOT NULL | UTC ISO timestamp |

### sensor_readings

| Column | Type | Constraints | Description |
| --- | --- | --- | --- |
| id | INTEGER | PK AUTOINCREMENT | Row ID |
| elevator_id | TEXT | FK → elevators.id | Which elevator |
| sensor_id | TEXT | NOT NULL | ES-VS-01 ¦ ES35-SW ¦ RW-ST01D |
| timestamp | TEXT | NOT NULL, INDEX | UTC ISO datetime — primary time axis |
| accel_rms_mg | REAL |  | ES-VS-01: acceleration RMS in mg |
| velocity_rms_mms | REAL |  | ES-VS-01: velocity RMS in mm/s |
| peak_accel_mg | REAL |  | ES-VS-01: peak acceleration |
| vib_temperature_c | REAL |  | ES-VS-01: onboard temperature |
| env_temperature_c | REAL |  | ES35-SW: ambient temperature |
| env_humidity_pct | REAL |  | ES35-SW: relative humidity %RH |
| load_kg | REAL |  | RW-ST01D: cabin load in kg |
| synced | INTEGER | DEFAULT 0 | 0=local only, 1=synced to cloud |

### inference_results

| Column | Type | Constraints | Description |
| --- | --- | --- | --- |
| id | INTEGER | PK AUTOINCREMENT |  |
| elevator_id | TEXT | FK → elevators |  |
| timestamp | TEXT | NOT NULL, INDEX | Time of inference |
| model_name | TEXT | NOT NULL | vibration_anomaly ¦ health_score ¦ overload |
| model_version | TEXT | NOT NULL | Semantic version, e.g. '1.2.0' |
| status | TEXT | NOT NULL | NORMAL ¦ WARNING ¦ CRITICAL ¦ OVERLOAD |
| confidence | REAL |  | 0.0–1.0 model confidence |
| health_score | REAL |  | 0–100 composite health score (health_score model only) |
| features_json | TEXT |  | JSON snapshot of input features for audit/retraining |
| synced | INTEGER | DEFAULT 0 |  |

### alerts

| Column | Type | Constraints | Description |
| --- | --- | --- | --- |
| id | INTEGER | PK AUTOINCREMENT |  |
| elevator_id | TEXT | FK |  |
| inference_id | INTEGER | FK → inference_results | Triggering inference row |
| alert_type | TEXT | NOT NULL | VIBRATION_HIGH ¦ TEMP_HIGH ¦ OVERLOAD ¦ HEALTH_LOW |
| severity | TEXT | NOT NULL | WARNING ¦ CRITICAL ¦ EMERGENCY |
| message | TEXT | NOT NULL | Human-readable alert message |
| sent_at | TEXT | NOT NULL | UTC ISO datetime alert was dispatched |
| channel | TEXT | NOT NULL | slack ¦ email ¦ sms |
| acknowledged | INTEGER | DEFAULT 0 | 1 when tech marks as reviewed |
| acknowledged_by | TEXT |  | Technician name or user ID |
| acknowledged_at | TEXT |  | UTC timestamp of acknowledgement |

### maintenance_schedule

| Column | Type | Constraints | Description |
| --- | --- | --- | --- |
| id | INTEGER | PK AUTOINCREMENT |  |
| elevator_id | TEXT | FK |  |
| recommended_date | TEXT | NOT NULL | ISO date — AI-recommended service date |
| urgency | TEXT | NOT NULL | routine ¦ soon (7d) ¦ urgent (24h) ¦ immediate |
| reason | TEXT | NOT NULL | e.g. 'Vibration anomaly sustained 48h, health_score < 40' |
| estimated_rul_hours | REAL |  | Remaining Useful Life estimate |
| status | TEXT | DEFAULT 'pending' | pending ¦ scheduled ¦ completed ¦ cancelled |
| completed_at | TEXT |  | Actual completion datetime |
| technician | TEXT |  | Assigned technician name |
| created_at | TEXT | NOT NULL | When AI generated this recommendation |

# 4. API Contract

All endpoints are served by FastAPI on the edge device (default port 8000). Full OpenAPI/Swagger docs are auto-generated at GET /docs. All responses use JSON. All timestamps are ISO 8601 UTC.

Authentication PoC: HTTP Basic Auth or API key header (X-API-Key). Production: JWT Bearer tokens with role-based access (admin ¦ technician ¦ viewer).

## 4.1  Sensor Data Endpoints

GET /api/elevators Returns list of all registered elevators with current status. Response 200: application/json Field; Type; Description. elevators; array[ElevatorObject]; List of elevators. elevator.id; string (UUID); Unique ID. elevator.name; string; Display name. elevator.status; NORMAL¦WARNING¦CRITICAL; Latest inference status. elevator.health_score; number 0–100; Latest health score. elevator.last_reading_at; ISO datetime; Timestamp of most recent sensor poll.

GET /api/elevators/{elevator_id}/readings Returns paginated raw sensor readings for a given elevator. Query Param; Type; Description. from; ISO datetime; Start of time range (default: now - 1h). to; ISO datetime; End of time range (default: now). sensor_id; string; Filter by sensor: ES-VS-01 ¦ ES35-SW ¦ RW-ST01D. limit; integer; Max rows returned (default 500, max 5000).

POST /api/predict (manual inference trigger) Accepts a JSON payload of sensor readings and returns immediate inference — useful for testing and one-off checks. Request Body Field; Type; Notes. elevator_id; string; Required. accel_rms_mg; number; From ES-VS-01. velocity_rms_mms; number; From ES-VS-01. peak_accel_mg; number; From ES-VS-01. vib_temperature_c; number; From ES-VS-01 onboard temp. env_temperature_c; number; From ES35-SW. env_humidity_pct; number; From ES35-SW. load_kg; number; From RW-ST01D. Response 200 Field; Type; Notes. status; NORMAL¦WARNING¦CRITICAL; Anomaly model output. confidence; number 0.0–1.0; Model confidence. health_score; number 0–100; Composite health score. features; object; Computed feature values for transparency. alert_triggered; boolean; Whether an alert was dispatched. model_version; string; ONNX model version used. inference_ms; number; Inference latency in milliseconds.

## 4.2  Alert & Maintenance Endpoints

| Method | Auth | Endpoint | Body / Params | Description |
| --- | --- | --- | --- | --- |
| GET | viewer | /api/alerts | ?elevator_id, severity, acknowledged, limit | Paginated alert history |
| PATCH | tech | /api/alerts/{id}/acknowledge | { technician, note } | Mark alert as reviewed |
| GET | viewer | /api/maintenance | ?elevator_id, status | Maintenance schedule list |
| POST | admin | /api/maintenance | { elevator_id, date, technician, note } | Manually create maintenance entry |
| PATCH | tech | /api/maintenance/{id} | { status, completed_at, notes } | Update maintenance status |
| GET | viewer | /api/elevators/{id}/health-history | ?from, to, granularity | Health score time series for charting |
| POST | admin | /api/models/reload | {} | Hot-reload ONNX models on edge device |
| GET | viewer | /api/health |  | System health check: DB, sensors, model status |

## 4.3  WebSocket — Live Sensor Stream

WS ws://edge-device:8000/ws/sensors/{elevator_id} Pushes a JSON message every poll cycle (configurable, default 5s) with the latest sensor readings and inference result. Used by the dashboard for live charts. WS Push Field; Type; Example. event; string; sensor_update. elevator_id; string; elev-001. timestamp; ISO datetime; 2025-04-15T08:23:11Z. readings; object; { accel_rms_mg: 42.3, load_kg: 320, ... }. inference; object; { status: 'WARNING', health_score: 61, confidence: 0.87 }. alert; object ¦ null; { type: 'VIBRATION_HIGH', severity: 'WARNING' } or null.

## 4.4  Error Responses

| HTTP Code | Error Code | When |
| --- | --- | --- |
| 400 | VALIDATION_ERROR | Request body fails Pydantic schema validation |
| 401 | UNAUTHORIZED | Missing or invalid API key / JWT |
| 403 | FORBIDDEN | Valid auth but insufficient role (e.g. viewer trying PATCH) |
| 404 | ELEVATOR_NOT_FOUND | elevator_id does not exist |
| 422 | SENSOR_DATA_INVALID | Readings outside expected physical range (e.g. accel_rms < 0) |
| 503 | SENSOR_UNAVAILABLE | Edge device cannot reach sensor bus (Modbus timeout) |
| 503 | MODEL_NOT_LOADED | ONNX model file missing or corrupt at startup |

# 5. Frontend Structure

Phase 1 (PoC) uses Streamlit for rapid development. Phase 2 (production) migrates to React + TypeScript with Recharts and shadcn/ui. The API contract in Section 4 remains identical for both — only the client changes.

## 5.1  Streamlit App (PoC)

| Page / Tab | File | Content |
| --- | --- | --- |
| Fleet Overview | pages/1_fleet.py | All elevators map, status badges (green/yellow/red), last reading time, health score gauges |
| Live Monitor | pages/2_live.py | Dropdown: select elevator → auto-refresh line charts (accel_rms, load_kg, temp) using st.empty + WebSocket or REST polling every 5s |
| Alert Inbox | pages/3_alerts.py | Filterable table of alerts, acknowledge button per row, severity colour coding |
| Maintenance Schedule | pages/4_maintenance.py | Calendar view + table, create manual entry form, mark complete |
| Model Performance | pages/5_models.py | Confusion matrix, AUC curve, feature importance (SHAP), model version info |
| Admin / Config | pages/6_admin.py | Sensor config editor (poll intervals, thresholds), model reload button, cloud sync status |

## 5.2  React App (Production) — Directory Structure

src/ api/ # Axios client, typed API hooks (useElevators, useAlerts...) components/ ui/ # shadcn/ui base components (Button, Badge, Card...) charts/ # SensorLineChart, HealthGauge, AlertTimeline elevator/ # ElevatorCard, StatusBadge, FleetGrid alerts/ # AlertTable, AcknowledgeModal maintenance/ # MaintenanceCalendar, ScheduleForm pages/ Dashboard.tsx # Fleet overview — all elevators at a glance LiveMonitor.tsx # Single elevator: live charts via WebSocket Alerts.tsx # Alert inbox with filters + acknowledge Maintenance.tsx # Schedule management ModelInsights.tsx # SHAP plots, model metrics, version history Admin.tsx # Thresholds, sensor config, API keys hooks/ useWebSocket.ts # WS connection with auto-reconnect useRolling.ts # Buffer last N readings for chart window store/ # Zustand global state (selected elevator, alerts) types/ # TypeScript interfaces (SensorReading, Alert...)

## 5.3  Key UI Component: Live Sensor Chart

The live monitor page connects to the WebSocket endpoint and maintains a rolling 60-point buffer. Each new message updates the chart state without full re-render:

| Component | Library | Notes |
| --- | --- | --- |
| SensorLineChart | Recharts <LineChart> | 3 lines: accel_rms, velocity_rms, peak; brush for zoom |
| HealthGauge | Recharts <RadialBarChart> | 0–100 arc, colour: green>70, yellow>40, red<40 |
| AlertTimeline | Custom <svg> or Recharts | Event markers on time axis, click for detail modal |
| LoadBar | Recharts <BarChart> | load_kg vs max_capacity, threshold line |
| StatusBadge | shadcn/ui <Badge> | NORMAL=green, WARNING=yellow, CRITICAL=red, pulsing animation |
| TempHumidityCard | Recharts <AreaChart> | Dual Y-axis: temp °C left, humidity %RH right |

# 6. Deployment & Configuration

## 6.1  Docker Compose (Edge Device)

version: '3.9' services: sensor_poller: build: ./services/sensor_poller devices: ['/dev/ttyUSB0:/dev/ttyUSB0'] # RS-485 adapter volumes: ['./data:/app/data'] environment: [ELEVATOR_ID=elev-001] restart: always inference_engine: build: ./services/inference volumes: ['./models:/app/models', './data:/app/data'] depends_on: [sensor_poller, redis] restart: always api_server: build: ./services/api ports: ['8000:8000'] volumes: ['./data:/app/data'] environment: - SECRET_KEY=${SECRET_KEY} - SLACK_WEBHOOK=${SLACK_WEBHOOK} - SMTP_HOST=${SMTP_HOST} restart: always redis: image: redis:7-alpine restart: always streamlit: # PoC dashboard — swap for Nginx+React in production build: ./dashboard ports: ['8501:8501'] restart: always

## 6.2  config.yaml — Edge Configuration

serial: port: /dev/ttyUSB0 baudrate: 9600 timeout_s: 1.0 sensors: vibration: { slave_id: 1, poll_interval_s: 5, model: ES-VS-01 } temp_humid: { slave_id: 2, poll_interval_s: 30, model: ES35-SW } load: { slave_id: 3, poll_interval_s: 1, model: RW-ST01D } elevator: id: elev-001 max_capacity_kg: 1000 thresholds: # Rule engine hard limits (pre-model alert triggers) accel_rms_warning_mg: 80 accel_rms_critical_mg: 150 load_overload_pct: 0.95 # 95% of max_capacity motor_temp_warning_c: 65 motor_temp_critical_c: 80 models: vibration_anomaly: models/vibration_anomaly_v1.onnx health_score: models/health_score_v1.onnx alerts: rate_limit_minutes: 15 # suppress duplicate alerts per type per elevator slack_webhook: ${SLACK_WEBHOOK} smtp_host: ${SMTP_HOST} smtp_port: 587 smtp_from: alerts@your-company.com smtp_to: [tech@your-company.com]

# 7. Implementation Checklist

## Phase 1A — Hardware & Connectivity (Week 1–2)

- Obtain RS-485 to USB adapter (isolated, e.g. EPCB USB-TO-RS485-ISOLATED)

- Assign unique Modbus Slave IDs to each sensor: VS-01=1, ES35-SW=2, RW-ST01D=3

- Wire sensors on shared RS-485 bus: A/B differential pair + GND

- Enable 120Ω termination resistor on ES35-SW (has built-in switch)

- Test each sensor with Insight Sensor software (Windows) to verify register map

- Document actual register addresses from ES-VS-01 and RW-ST01D datasheets

- Verify HD-MV01A loadcell is calibrated and connected to RW-ST01D correctly

- Confirm Orange Pi 4 Pro can read all 3 sensors via minimalmodbus

## Phase 1B — Data Pipeline (Week 2–4)

- Implement sensor_poller.py with per-sensor threading and exponential backoff

- Create SQLite schema (Section 3) with proper indexes on (elevator_id, timestamp)

- Collect 48h of baseline data from all sensors — elevator running normally

- Implement preprocessor.py with rolling window features (Section 2.3)

- Validate feature distributions match expectations (no NaN, no extreme outliers)

- Set up Redis as inter-process queue between poller and inference engine

## Phase 1C — ML Models (Week 4–8)

- Train Isolation Forest on 48h baseline vibration features — export as ONNX

- Label first anomaly events manually: inject load spike, simulate high vibration

- Train XGBoost classifier on labeled data (NORMAL / WARNING / CRITICAL)

- Train LSTM Autoencoder on multivariate sensor sequence (10-min windows)

- Implement health_score = weighted combo of anomaly confidence + trend score

- Implement rule-based overload detection (load_kg > 95% capacity)

- Implement RUL estimation via linear regression on 7-day health_score trend

- Export all models to ONNX, benchmark inference time on target edge device

## Phase 1D — API & Dashboard (Week 8–12)

- Implement FastAPI server with all endpoints from Section 4

- Implement WebSocket /ws/sensors/{elevator_id} with 5s push cadence

- Implement alert_dispatcher with Slack webhook + SMTP + rate limiting

- Build Streamlit dashboard: Fleet Overview, Live Monitor, Alert Inbox, Maintenance

- Deploy all services via docker-compose on edge device

- Set up cloud sync job to PostgreSQL (Supabase or AWS RDS free tier)

- End-to-end test: simulate anomaly → inference → alert → dashboard update

- Load test API: 10 concurrent dashboard clients, verify < 300ms p95 latency

END OF DOCUMENT — Elevator Predictive Maintenance Implementation Plan v1.0